# Single Board Computer for 6502 Microprocessors

# Single Board Computer for 6502 Microprocessors (SBC6502)

## [Overview](README_Overview.md) &nbsp;&nbsp; <sub>[SBC6502 Hardware](README_SBC6502.md)</sub> &nbsp;&nbsp; <sub>[Virtual Memory Kernel (VMK)](README_VMK.md)</sub>

*A lifelong learning experience ...*

<br/>

The *Single Board Computer for 6502 Microprocessors* is my amateur project to explore and understand the computer-science around multi-tasking operating systems and virtual memory hardware.  The 6502 processor was chosen for its software and hardware simplicity (and challenge.)  And as a nod to the KIM-1 used in an undergraduate term assignment.  All design decisions lean towards simplicity and implementing minimum needed to research the stated topics using small to medium scale integration (7400 series logic).

### Virtual Memory Kernel (VMK)

•   Multi-Tasking – More than one active process in memory at a time.

•   Preemptive Tasking – Processes are blocked to allow other processes access to computer resources.

•   Kernel Mode – Processes may run in an unprotected mode allowing access to all computing resources.

•   User Mode – Processes may run in protected mode protecting computing resources from other processes.

•   Virtual Memory – Each process sees a continuous 64KB memory environment.

### Memory Management Unit (MMU)

•   A configurable MMU allowing variable address bus widths and page sizes.

•   Up to a 24-bit 4294MB address space shared by RAM, ROM and memory mapped I/O.

•   Page sizes vary from 512 bytes thru 4KB.

•   All primary control logic (MMU, memory decoding, etc.) will be implemented with 7400 series ICs.

•   Arduinos may be used to implement complex I/O devices such as video, mass storage and serial communication.

## Overview

The 6502 is an 8-bit processor with a16-bit 65KB memory model.  Sections of the address space are proportioned off for the Zero Page (256 bytes), Stack (256 bytes) and interrupt vectors (6 bytes).  The goal of the VKM and MMU will be to present the full 64KB memory model to each process.  The MMU’s Configuration Registers (CR0 & CR1) are programmable at boot time and determine how the MMU will function.

The MMU has 2 memory-mapped configuration registers that control:

* The number of effective memory bits; the total address space of the SBC6502

* The number of bits in a Process ID (PID)

* The size of each memory page

* The number of process bits used to address a page segment

The MMU provides a protected Kernal Mode by intercepting the BRK and RTI instructions.

```

R0 - Mode Register (0x0200)

┌─┬─┬───────────┐

│7 6 5 4 3 2 1 0│

└┬┴┬┴─────┬─────┘

│ │      └───────┤ PID (1-6 bits)

│ │

│ └──────────────┤ ~Kernal Mode (0 is enabled) / User Mode (1 is enabled)

│

└────────────────┤ ~Boot Mode (0 is enabled) / Address Translate Mode (1 is enanled)

R1 - Status Register (0x0201)

┌─┬─┬─┬─┬─┬─┬─┬─┐

│7 6 5 4 3 2 1 0│

└┬┴┬┴┬┴┬┴┬┴┬┴┬┴┬┘

│ │ │ │ │ │ │ └──┤ Boot Address Error

CR0 - Config Register 0 (0x0202)

┌───────┬───────┐

│7 6 5 4 3 2 1 0│

└───┬───┴───┬───┘

│       └─────┤ # Page Offset Bits (0-15 bits)

│

└─────────────┤ # Page Segment Bits (0-15 bits)

CR1 - Config Register 1 (0x0203)

┌───────────────┐

│7 6 5 4 3 2 1 0│

└──┬──┴────┬────┘

│       └──────┤ # Effective Address Bits (0-31 bits)

│

└──────────────┤ # PID Bits (0-6 bits)

MVR(s) - Memory Vector Register(s)  (0xFFFF - 0xFFFF)

┌───────────────┐

│7 6 5 4 3 2 1 0│─┤ MVRlow (0xFFEE)

└───────────────┘

.               .

┌───────────────┐               .

│7 6 5 4 3 2 1 0│─────────────────┤ MVRhigh (0xFFEF)

└───────────────┘               .

.               .               .

┌─┬─┬─┬─────────────────────────┐

│5 4 3 2 1 0 9 8 7 6 5 4 3 2 1 0│

└┬┴┬┴┬┴────────────┬────────────┘

│ │ │             └──────────────┤ Upper Effective Address Bits (0-13 bits)

│ │ │

│ │ └────────────────────────────┤ Vector Active

│ │

│ └──────────────────────────────┤ Write Enable

│

└────────────────────────────────┤ Page Dirty

MVP - Memory Vector Pointer

┌───────┬───────────────────────┐

│5 4 3 2 1 0 9 8 7 6 5 4 3 2 1 0│

└───────┴───────────┬───────────┘

```