README_Overview.md

The *Single Board Computer for 6502 Microprocessors** is my amateur project to explore and understand the computer-science around multi-tasking operating systems and virtual memory hardware.  The 6502 processor was chosen for its software and hardware simplicity (and challenges.)  And as a nod to the KIM-1 used in an undergraduate project assignment.  All design decisions lean towards simplicity and implementing the minimum needed to study the stated topics using small to medium scale integration (7400 series logic).

> *Click bait - after thinking about the goals of this project, I scraped the idea (at this time) of a single board computer.  I may want to swap out the virtual memory hardware to try out different solutions, for example.  And the projected chip count of the memory management logic necessitates the use of a computer backplane design with modular logic cards.  A backplane allows the replacement of logic sections until the final design is hammered out.  Although “SBC6502” is a misnomer, it will continue to be used as the project name (and GitHub repo name.)

Other general goals and requirements:

* All control logic will be implemented with Small Scale Integration (SSI) 7400 series ICs.  (The 74610, 611, 612 or 631 ICs will not be used.)

* However, LSI devices (and Arduinos) may be used for complex functions such as video, mass storage and serial communication.

Ultimately this is both a hardware and software project.  Yes, hardware is needed to map the 6502’s physical 16-bit address space into a 24-bit virtual memory address space.  But software is required to manage and monitor virtual memory as processes are executed by the 6502 microprocessor.

The Memory Management Unit (MMU) provides architectural enhancements to the 6502 that the Virtual Memory Kernel (VMK) relies on to meet its required goals.    VMK requirements are listed here to remind me of the what the MMU must support in the end.  [VMK6502](https://github.com/craigory1958/VMK6502.git) is a separate GitHub project supporting the development of the Virtual Memory Kernel.

README_SBC6502.md

The 6502 is an 8-bit processor from the 70's with a 16-bit 65KB memory model. The address space is proportioned off for the Zero Page (256 bytes), Stack (256 bytes) and interrupt vectors (6 bytes) and uses memory mapped I/O. The goal of this project is to develop the hardware to support a full fledge preemptive multi-tasking kernel that presents each process with the full 6502 64KB address space.

Why a 6502 and not a more modern, more powerful microprocessor?  I want to study the hardware (and software) requirements of providing a modern computer system. I also have fond memories of owning a KIM-1 and using it in my undergraduate studies. The 6502 is my favorite microprocessor.

Rather than trying to develop a single large PCB board implementing all functions of the SBC6502 computer, a backplane will be used. Functions of the SBC6502 such as MMU, ROM/RAM, and I/O can be debugged independently as modular logic boards slotted into the backplane.

README_SBC6502_Backplane.md

Because of the projected number of ICs needed and the difficulty of developing a single monolithic PCB, an organization of modular logic cards plugged into a backplane was chosen. Modularity allows the development and debugging of sub-functions, like memory or clock circuitry without having to re-produce a monolithic PCB.

The RCBus standard developed by [Small Computer Central](https://smallcomputercentral.com/) for homebrew 8-bit retro computing provides a turnkey backplane that will work for this project.  They offer 6 slot backplane kit [SC701 Backplane Kit for RCBus](https://www.tindie.com/products/tindiescx/sc701-backplane-kit-for-rcbus-80pin/).  The RCBus specification was developed for Z80 based microprocessors. It is basically an 80-line bus, some lines will be repurposed for a 6502 based project.  Some lines will not be used.  The only short fall of renaming some of the bus lines is that they will be misnamed on the backplane’s silkscreen.

<img src="_assets/images/sc701-assembled-c-w600.jpg">

| Header 1 | Header 2 | Header 3 |
|---|---|---|
| Row 1 Col 1 | Row 1 Col 2 | Row 1 Col 3 |
| Row 2 Col 1 | Row 2 Col 2 | Row 2 Col 3 |

debugged independently as modular logic boards slotted into the backplane.

README_SBC6502_MemoryManagmentUnit.md

The VMK interacts with the MMU thru 4 8-bit memory mapped I/O registers

* R0 - Mode Register (0x00200)

```

┌─┬─┬───────────┐

│7 6 5 4 3 2 1 0│

└┬┴┬┴─────┬─────┘

│ │      └───────┤ PID (1-6 bits)

│ ├

│ └──────────────┤ ~Kernel Mode (0 is enabled) / User Mode (1 is enabled)

│

└────────────────┤ ~Boot Mode (0 is enabled) / Address Translate Mode (1 is enabled)

```

The MMU’s Configuration Registers (CR0 & CR1) are programmable at boot time and determine how the MMU will function.

The MMU has 2 memory-mapped configuration registers that control:

* The number of effective memory bits; the total address space of the SBC6502

* The number of bits in a Process ID (PID)

* The size of each memory page

* The number of process bits used to address a page segment

The MMU provides a protected Kernal Mode by intercepting the BRK and RTI instructions.